How to use your Leaf Gauge

A leaf gauge is a very simple device similar to a "feeler gauge" except all leaves are of the same dimension.  Leaves may be of a mylar, polyester, polycarbonate, acetate or other type of plastic material.  Ideally they should bend freely without creasing & rebound to their original shape for reuse.

Long introduced the leaf gauge in 1973, at which time he discussed its construction & use in the locating of centric relation & in adjusting occlusions.  Williamson et al further elaborated its clinical applications in determining & recording centric relation.  Regenos & myself have been teaching the use of the leaf gauge since the mid-seventies.  Shankland & Ralston reported the use of the leaf gauge & our teaching philosophy in 1983.  Golsen & Shaw reported how to make a leaf gauge along with its use in 1984.  Rosenblum conceived the idea of numbering the leaves & reported a construction technique along with Huffman in 1985.  McHorris advocates the use of a leaf gauge by MPDS patients to cycle their muscle spasm symptoms.  Regenos uses the leaf gauge exclusively when trying-in & seating castings.  Huffman reported his cusp-fossae equilibration technique utilizing a numbered leaf gauge in 1987. Carroll et al reported on the use of an anterior jig or leaf gauge in routine clinical practice.

The leaf gauge is held by the dentist or assistant with "X" number of leaves placed into the oral cavity at the midline, resting on (& parallel to) the lingual surfaces of the maxillary central incisor teeth.  The patient is instructed to close on his (or her) back teeth until a lower front tooth touches the underside of the leaves.  "X" number of leaves is that number where the patient can just barely feel a posterior tooth touch (arrived at by the trial & error addition of leaves).  With X+1 leaves the patient initially cannot feel any posterior tooth contact.  However, as he or she holds their jaw closed (with only half hard closing force) for approximately 15-20 seconds they will again be able to feel a posterior tooth contact.  At this time the mandible is said to be tripodized in its centric relation position with the right & left condyles seated in their true, respective glenoid fossae housings with only a lower incisor tooth making contact on the underside of the leaves.

This eliminates potential operator guided errors in centric relation by permitting the patients’ own neuromusculature to seat the condyles into their centric relation positions.  The average is approximately X+6 leaves to arrive at this finalized position with a normal range of X+2 to X+9 leaves.

If you are using the leaf gauge for an occlusal equilibration or to adjust castings to centric relation occlusion you start with the finalized X+? number of leaves.  Gradually remove leaves one at a time, marking the prematurity each time with mylar tape such as Accufilm II or MDSII & making the necessary adjustments as each leaf is removed.

When utilizing the leaf gauge to make centric relation interocclusal records for mounting diagnostic casts, the operator starts with the finalized X+? leaves & arbitrarily adds 3 or 4 additional leaves as a "fudge" factor so the teeth cannot close completely through the recording material (which would result in an inaccurate record).  An anterior cut-out is made in the recording wax or the framework to house the leaves.  The recording material is positioned onto the maxillary arch of teeth, the leaf gauge properly inserted and the patient instructed to close as before until the lower front tooth touches the leaf gauge; gently but firmly hold the mandible in this position until the recording material chills or sets, as the case may be.  We recommend cloth reinforced Aluwax bite wafers for the recording material for centric relation interocclusal records to mount diagnostic casts (master treatment casts require more accurate materials); however, you may use the recording material of your choice as long as a voided space is provided for the placement of the leaves.

I suggest you consult Dr McHorris for leaf gauge utilization for the physical therapy technique to cycle painful muscle spasm.  At: 3100 Walnut Grove, #302; Memphis, TN 38111

This numbered leaf gauge permits easy recording of the exact number of leaves.  Also, since the leaves are almost exactly 0.1mm thick it gives the operator a very rapid reading of the exact opening in mm's (eg. 5 leaves equals 0.5mm opening).

CAUTION: It has been reported in the literature that if the patient is permitted to close with maximum closing force on the leaf gauge it is possible to displace the mandible slightly inferior & posterior.  This has never been a problem since we merely caution the patient to close only about half as hard as they can.

!!!!ENJOY YOUR LEAF GAUGE!!!!

FREQUENTLY ASKED QUESTIONS

Q	How do I take care of the Huffman Leaf Gauge?

DO NOT autoclave the gauge as this can cause damage.  DO disinfect if gluteraldehyde cold sterilisation solutions or wipe with alcohol.  It works best to fan the leaves while cleaning & drying to prevent them sticking together.

Q	How long will the Leaf Gauge last?

The gauge will last through many uses & disinfections.  The exact number of uses is indeterminable due the differences in users & uses.  Your satisfaction is guaranteed.